Whitton and Tosson Parish Council Highways and Footpaths Report

Date: 	07 May 2019

Highways Inspection:

- The road surface to the west of the entrance to Whitton Glebe Caravan Park is rough and in need of a new surface for several metres.

- There are two sections of road surface adjacent to Tosson Glebe Farm that need a new surface

- There are 4 potholes in the road down past Tosson Mill that ends at Weaver’s cottages. This work was missed in the pot hole repair session following our report 05 February.

- The passing place at the bend on the road from Newtown to Tosson needs 3 to 4 ton of crushed stone to fill in the deep gash at the other side of the road.

- There are two potholes near the seat on the Tosson to Lordenshaws road.

- There are two potholes at the side of the road beyond the entrance to West Newtown Farm.

Footpaths:

- Two way markers need replacing on the Simonside to Whitton Hillhead bridleway .

- Planting of oak and silver birch trees has been completed on two triangular sections of derelict ground on Carterside road. The trees are well away from the highway and the overhead Broadband cable.

- Heaps of salt on Tosson to Lordenshaws road – what should be done with them?

Verges:

Seats & Road Signs:

- We have identified a road signage issue for Silverton Lane to be included in the next LTP proposals. (Possible signage for ‘Single Track Road with Passing Places’ and ‘Not suitable for HGV’s’)

Drains:

- Drain at entrance to Lime Kiln car park checked and found to be serviceable.

- Drain under Carterside road checked – it is in good working order with settlement sump upside of pipe.

- The ditch on the road from Newtown to Tosson has been checked. It is in need of cleaning out as there is a large quantity of hedge cuttings/sand/gravel filling it in places with some of the entrances to the piped sections buried/lost. These have been dug out by hand but will soon be blocked again.

Signed: Alan Winlow